Блок-схема алгоритма

Рис.1 Основной блок программы main

Рис.2 Функция get_real_time

Рис.3 Функция interrupt my_timer

Текст программы

#include <dos.h>

#include <stdio.h>

#include <conio.h>

void interrupt(*sys_timer)();

void interrupt my_timer();

unsigned long count = 0;

unsigned long saved_times[10];

unsigned char saved_count = 0;

unsigned char timer_running = 1;

void get_real_time(unsigned char* h, unsigned char* m, unsigned char* s) {

union REGS r;

r.h.ah = 0x2C;

int86(0x21, &r, &r);

*h = r.h.ch;

*m = r.h.cl;

*s = r.h.dh;

}

void interrupt my_timer() {

if (timer_running) {

count++;

}

sys_timer();

}

int main() {

unsigned char hours, minutes, seconds;

char key = ' ';

int i;

clrscr();

sys_timer = getvect(0x08);

setvect(0x08, my_timer);

printf("Timer: S-Start P-Pause R-Reset T-Save C-Clear Q-Quit\n");

while (1) {

if (kbhit()) {

key = getch();

switch (toupper(key)) {

case 'S': timer_running = 1; break;

case 'P': timer_running = 0; break;

case 'R': count = 0; break;

case 'T':

if (saved_count < 10) saved_times[saved_count++] = count;

break;

case 'C': saved_count = 0; break;

case 'Q':

setvect(0x08, sys_timer);

return 0;

}

}

get_real_time(&hours, &minutes, &seconds);

gotoxy(1, 3);

printf("Time: %02u:%02u:%02u  Ticks: %-8lu  State: %-7s",

hours, minutes, seconds, count,

timer_running ? "RUNNING" : "PAUSED");

gotoxy(1, 4);

printf("Elapsed: %-6.1fsec \nSaved: %d/10", count / 18.2, saved_count);

gotoxy(1, 6);

printf("Last command: %c", key);

gotoxy(1, 7);

printf("Saved times:                                  ");

for (i = 0; i < saved_count; i++) {

gotoxy(1, 8 + i);

printf("%d. %-6lu ticks (%-6.1f sec)           ",

i + 1, saved_times[i], saved_times[i] / 18.2);

}

for (i = saved_count; i < 10; i++) {

gotoxy(1, 8 + i);

printf("                                    ");

}

delay(50);

}

setvect(0x08, sys_timer);

return 0;

}

Примеры запуска программы

Рис.4 Работа программы

Заключение

Разработана программа обработки прерывания таймера IRQ0 в DOS. Реализована цепочка обработчиков: пользовательская функция подсчета тиков выполняется перед системным обработчиком. Обеспечена корректная работа с векторами прерываний. Программа демонстрирует низкоуровневое программирование и может служить основой для систем реального времени.